Appendix E

Intruder Probabilistic Models

# E.1	Linear Intersection

Idea: There are small intruders which have body smaller than average celli,j,k cell size. Its trajectory will stick to linear trajectory prediction with high probability.

Space Intersection Rate: The Space Intersection Rate for celli,j,k is implemented as simple point cloud intersection. Where sufficiently thick point cloud is defined along line (eq. E.1):

position(time) = position(time0) + velocity × time,	time ∈ [0,∞[	(E.1)

Then there exist projection function from local Euclidean coordinates to local polar coordinates (eq. E.2. The function projects intruder trajectory (eq. E.1) to planar coordinates [distance, horizontal◦, vertical◦] as a set of sufficiently thick point cloud.

polarSet : position(t) → {[distance,horizontal◦],vertical◦}	(E.2)

The space intersection rating SpaceIntersection(◦) for line type is given as (eq. E.3). If there exist non empty intersection of polarSet ∩ celli,j,k there is space intersection rate equal to 1, if intersection polarSet ∩ celli,j,k = ∅ then the rate is zero.



!

Intruder, 1 :	∃point ∈ polarSet(eq.E.2) : point ∈ ci,j,k

space	=	(E.3)

cell,j,k	0 :	otherwise

Note. The intruder intersection rate is multiplication of space intersection rate and time intersection rate. The intersection rate is calculated for every intruder and selected intersection model separately.

1

# E.2	Body-volume Intersection

Idea: The Intruder has body volume greater than average celli,j,k volume. The intruder body is considered as the ball moving along intruder position. The intersection of the intruder body is realized as sufficiently thick point-cloud intersection.

Space Intersection Rate - Body Volume: The body volume mass with center at position(t) is moving along intruder trajectory prediction (eq. E.4) in time interval [0,∞[:

position(time) = position(time0) + velocity × time	(E.4)

The body Volume ball Body(position(t),radius) (eq. E.5) is defined as set of points in R3 euclidean space. The center is moving along the position(t). The body volume ball is a set of points sufficiently thick including also inner points. The thickness is guaranteed by existence of neighbour point which is close enough.

	 kposition(t) − point ≤ radius

	3	

Body(position(t),radius) =	point ∈ R : ∀pointi∃pointj6=i,

	distance(pointi,pointj) ≤ thickness

(E.5) The polar volume ball polarBody (eq. E.6) is projection of body volume ball set Body(position(t),radius) to a set of planar coordinates in avoidance grid coordinate frame:

The space intersection rate for vehicle body space(Intruder,celli,j,k) (eq. E.7) is calculated as intersection of polar body volume ball and celli,j,k. If intersection is non empty then base probability is one, zero otherwise:



!

Intruder, 1 :	∃point ∈ polarBall(eq.E.6) : point ∈ ci,j,k

space	=	(E.7)

celli,j,k	0 :	otherwise

Intersection Time: The intersection time id depending on point cloud (eq. E.6) where each point have intersection time given as body-center position time (eq. E.4).

Note. The body-volume intersection model can insert the multiple intersection times into one celli,j,k. The interval length considers all of these for intersection rates (eq. ??).

# E.3	Maneuverability Uncertainty Intersection

Idea: The intruders are not bullets they are not sticking to predicted linear paths. The intruder maneuverability is given as horizontal and vertical spread. Therefore intruder reach set will form an elliptic cone. This cone can be transformed into finite discrete point-cloud, each point should have assigned severity impact value. The point cloud intersection with Avoidance Grid will give us space impact of an uncertain intruder.

Note. The following section will use condensed notation, due to the equation complexity. The terminology is consistent with the rest of the section.

Space Intersection Rate - Body Volume Intersection: PT (ik(xs,v,θ,ϕ),ci,j,k) computation is less straight-forward than other space intersection rates. First let us define the linear intruder ik positions x at time t (eq. E.8) model, where x(t) defines intruder position in avoidance grid euclidean coordinate frame at time ti, v defines intruder velocity, and t is a time offset.

x(t) = xs + vI.t	(E.8)

Intruder horizontal spread θ and vertical spread ϕ are introduced. These spreads represents intruder deviation limits along from linear trajectory prediction x(t) ∈ R3. The example is given by (fig. E.1) where the intruder starts at point xs with fixed velocity v, the linear trajectory prediction is outlined by blue line. The predicted intruder position at time t = 10s is given by x(10) (blue point). The ellipsoidal space E(x) is projected on the plane D(x(t)). The plane D (eq. E.9) for point x(t) and velocity v is defined as an orthogonal plane to velocity vector v ∈ R3 with origin at intruder position x(t).

(E.9)

To construct ellipsoidal space boundary on orthogonal plane D(x(t),v) some parameters are defined in (eq. E.10). The scalar distance ddx(t) is simple Euclidean norm, maximal horizontal offset dθ(xt) is given as product of sinus of horizontal offset angle θ and scalar distance dd, and maximal vertical offset dϕ(x(t)) is given a product of sinus of vertical offset angle ϕ and scalar distance dd.

dd = dd(x(t),xs) = kx(t) − xsk2

dθmax = dθ(x(t)) = sinθ(ik).dd(x(t))	(E.10)

dϕmax = dϕ(x(t)) = sinϕ(ik).dd(x(t))

The Ellipsoid E(x(t),v) (eq. E.11) for fixed intruder position x(t) and fixed intruder velocity v is given as constrained portion of orthogonal plane D(x(t),v). The constraint is defined by an internal coordinate frame p ∈ R2 which is space reduction of plane D(x(t),v).

Figure E.1: One rate position [dd,dθ,dϕ] (green). deviated from linear trajectory (blue line) at point x(10).(blue) with initial position xs (red)

The internal coordinate frame p ∈ R2 has origin in x(t) → R2. The points of plane p are bounded by projection p = (b − x(t)) → R2, where b ∈ D(x(t),v). The point of ellipsoidal p is then given as standard ellipse boundary with vertical span dθ(x(t)) and horizontal span dϕ(x(t)).

The 2D Ellipsoid E(x(t),v) for specific time t = 10s example is portrayed as red ellipsoid (in fig. E.1).

(E.11)

The expected behavior of an intruder ik is to stick to predicted linear trajectory x(t) (E.8). The probability of deviation should be decreasing with distance from the ellipse center (fig. E.2.).

Probability density function for ellipsoid E(x(t),v)defined in (eq. E.11) is depending on maximal horizontal spread dθ(x(t)), maximal vertical spread dϕ(x(t)), defined by (eq. E.10).

Two standard probabilistic distributions are established N(µθ,σθ) (eq. E.12) for horizontal spread θ(x(t)) and N(µϕ,σϕ) (eq. E.13) for vertical spread ϕ(x(t)). The means µθ and µϕ are set to zero, and internal coordinate frame p ∈ R2 where x(t) → R2 is frame center. The variances σθ and σϕ are set as maximal distances on horizontal/vertical spread axes dθ(x(t)) and dϕ(x(t)).

P(x(t),dθ) = N(µθ,σθ) = N(0,dθ(x(t)))	(E.12)

P(x(t),dϕ) = N(µϕ,σϕ) = N(0,dϕ(x(t)))	(E.13)

The combined probability density function for maximal spreads dθ and dϕ is given by (eq. E.14). Because probability density function is defined for internal space p ∈ R2 and one may need to calculate impact rate for cell space ci,j,k ∈ R3.

The reduction from two parameter probability distribution function to scalar rate distribution function is needed. A scalar rate distribution function P(x(t),dθ,dϕ) over ellipsoid E(x(t),v) is defined as (eq.E.14), where the final rate is given as an average of two partial probabilities.

Final space intersection rate P(x(t),dθ,dϕ) needs to be normalized to hold normal distribution condition (eq. E.15). Normal distribution condition value (eq. E.15) is given as surface integral over ellipsoid E(x(0),v) with rate distribution function P(x(t),dθ,dϕ).  (E.14)

ZZ

P(x(t),dθ,dϕ)ddθ ddϕ = 1	(E.15)

E(x(τ))

Final space intersection rate P(x(t),ci,j,k,θ,ϕ) (space portion, time portion is calculated in (eq.??) is given by (eq. E.17). Its mean value of all intersection rates P(x(τ),ci,j,k,θ,ϕ) where τ ∈ [ie(ci,j,k),il(ci,j,k)] is fixed point in intersection time interval.

An P(x(τ),ci,j,k,θ,ϕ) (E.16) is integration of rate density function P(x(τ),dθ,dϕ) (eq. E.14) in surface E(x(τ),v) to cell ci,j,k volume intersection.

Figure E.2: Probability of intruder ik position in ellipsoid E(x(t),v)

To get a volume integration partial rate in surface intersection must be integrated and normalized in time interval τ ∈ [ie(ci,j,k),il(ci,j,k)], the base intersection probability PT (ik(xs,v,θ,ϕ),ci,j,k) is given by (eq. E.17). Example of intersection of intruder ir uncertain ellipsoid cone with avoidance grid A(ti) is given in (fig. E.3).

ZZ

| P(x(τ),ci,j,k,θ,ϕ) = | P(x(τ),dθ,dϕ) E(x(τ),v)∩ci,j,k | (E.16) |
| --- | --- | --- |

(E.17)

Figure E.3: Avoidance grid A(ti) (blue) intersection with elliptic cone intruder ik(x,v,θ,ϕ) (red) example.

A numeric approximation of space intersection rate PT (ik(xs,v,θ,ϕ),ci,j,k) is more implementation feasible than symbolic calculation due to the multiple intersection constraints and bad intersection algorithm complexity.

Let us define a homogeneous discrete subset of real numbers R which is a non empty subset of real numbers R. The set R (eq. E.18) is homogeneous that means for an equal interval (i,i + 1],i ∈ Z subset the count of members is equal to some positive natural number k. The parameter k can be understood as unit approximation density.

Similarly, the power sets R2 ⊂ R2, R3 ⊂ R3, ... Ri ⊂ Ri,i ∈ N+ keeps homogeneous distribution.

R	(E.18)

The orthogonal plane for x(t),v,t ∈ R is defined by (eq. E.9). The orthogonality property is also kept for any subspace Rn ∈ Rn,n ∈ N+. Numeric approximation of D(x(t),v) is given as DD(x(t),v) (eq. E.19).

The only difference is that discrete approximation is countable |DD| = m,m ∈ N+, but continuous representation |D| ≈ ∞ is uncountable. Because ellipsoid is a subset of orthogonal plane it keeps its countability property; therefore ED is also countable and must contain at least one member.

R	(E.19)

The base ellipsoid E(x(t),v) for continuous-space is given by (eq. E.11). Every element, expect the base of internal projection R2 and orthogonal plane DD is same in discrete case ED(x(t),v) (eq. E.20).

R	(E.20)

The numeric calculation disproportion can occur in case that ellipsoid E¯D(x(t),v) (E.20) in case of dθ(x(t)) ≈ 0 and dϕ(x(t)) ≈ 0. The count of ellipsoid members can be |E¯D(x(t),v)| = 0, which is in contradiction with assumption |E¯D(x(t),v)| 6= 0.

Let assume for discrete times τ = {t1,t2,...,ti}, i ∈ N+ there exists ellipsoids E¯D(x(t1),v),E¯D(x(t1),v), ..., E¯D(x(ti),v) which are non empty and in space R2 in internal coordinate frame and space R3 in avoidance grid A(ti) coordinate frame. The intersection of these partial ellipsoids in both spaces is equal to:

E¯D(x(t1),v) ∩ E¯D(x(t2),v)··· ∩ ...E¯D(x(ti),v) = ∅	(E.21)

An empty intersection enables us to keep homogeneity property of ellipsoids by adding points so it is safe to add specific point x(t) into empty ellipsoid. But only one, because it does not impact probability density functions N(µθ,σθ) and N(µϕ,σϕ), neither space intersection rate density function P(x,dθ,dϕ).

The final ellipsoid used forward ED(x(t),v)(eq. E.22) is keeping all properties of ellipsoid E(x(t),v) (eq. E.22).

(E.22)

The normal distribution condition for rate distribution function PD(x(t),dθ,dϕ,p), which is instance of to rate density function P(x(y),dθ,dϕ) (eq. E.14) is used. This rate distribution must be normalized according to (eq. E.23).

X	+

PD(x(t),dθ,dϕ,p) = 1,∀t ∈ R	(E.23)

p∈ED(x(t))

The equations for space intersection rate are similar to (eq. E.16, E.17). For cell ci,j,k there exist intruder entry time ie(ci,j,k) its the earliest intersection with ellipsoid ED(x(ie(ci,j,k))),v. Same situation occurs with intruder leave time il(ci,j,k). Because ED is countable set, it means additional attributes can be attached to each point p ∈ ED.

Based on system dynamic (eq. ??) the Time Of Arrival (TOA) can be calculated. The example of TOA is given in fig. E.4.

Figure E.4: Time Of Arrival (TOA) for one ellipsoid ED(x(τ),v).

The intersection rate PD(x(τ),ci,j,k,θ,ϕ) for one time sample τ is given by (eq. E.24), which has similar notation to (eq. E.16), sums are used instead of integrals and discrete rate density function PD(x(τ),dθ,dϕ,p) for points form ellipse and cell intersection are used as iterator base set p ∈ {ED(x(τ),v) ∩ ci,j,k}.

PD(x(τ),ci,j,k,θ,ϕ) =	X	PD(x(τ),dθ,dϕ,p)	(E.24)

p∈{ED(x(τ),v)∩ci,j,k}

The space intersection rate PTD(ik(xs,v,θ,ϕ),ci,j,k) (eq. E.25) is given as mean intersection rate of partial intersections PD(x(τ),ci,j,k,θ,ϕ) where step set T = { ie(ci,j,k), ..., il(ci,j,k)} contains all viable intersection times with ellipsoids E(x(τ ∈ T),v). The denominator is basically count of samples in sample time set T.

(E.25)

An intersection of intruder cone and cell ci,j,k cell is defined by (eq. E.26) The set of point p ∈ R3 where condition of intersection between ellipsoids ED(x(τ),v) for times τ ∈ R+ and cell space ci,j,k is met.

P	(E.26)

An intruder time of entry ie(ik,ci,j,k) (eq. E.27), for intruder i,k and cell ci,j,k is approximated for discrete point set P(ik(xs,v,θ,ϕ),ci,j,k) (eq. E.26) as minimal time of arrival tTOA(p) of member points p.

ie(ik,ci,j,k) ≈ min{tTOA(p) : p ∈ P(ik(xs,v,θ,ϕ),ci,j,k)}	(E.27)

An intruder time of leave il(ik,ci,j,k) (eq. E.28), for intruder i,k and cell ci,j,k is approximated for discrete point set P(ik(xs,v,θ,ϕ),ci,j,k) (eq. E.26) as maximal time of arrival tTOA(p) of member points p.

il(ik,ci,j,k) ≈ max{tTOA(p) : p ∈ P(ik(xs,v,θ,ϕ),ci,j,k)}	(E.28)

Combined intersection model: The combined intersection model POI(ik,ci,j,k,l,b,s,τ) is defined for intruder ik with parameters:

- Starting position xs - expected position of intruder ir in 3D space at time of avoidance ti in avoidance grid frame A(ti).

- Velocity vector v - oriented velocity of intruder ir at time of avoidance ti in avoidance grid frame A(ti).

- Horizontal uncertainty spread θ - defines how much can intruder ir deviate on horizontal axis of intruder local coordinate frame (if X+ is the main axis, then Y is horizontal axis in right-hand euclidean coordinate frame), due the properties of intersection definition, the horizontal uncertainty spread can have following values θ ∈ [0,π/2].

- Vertical uncertainty spread ϕ -defines how much can intruder ir deviate on vertical axis of intruder local coordinate frame (if X+ is the main axis in local right-hand euclidean intruder coordinate frame, then Z is horizontal-vertical axis), due the intersection definition, the vertical uncertainty spread can have following values ϕ ∈ [0,π/2].

- Body volume radius r - defines the body volume of an intruder in meters and it has R+ value.

The flag vector l,b,s,τ ∈ {0,1} is a parametrization of rate calculation: l stands for the lined intersection, b stands for body intersection, s stands for the spread intersection, τ stands for time account.

The space intersection for line PL(ik,ci,j,k) is defined as PT (ik(x,v),ci,j,k), where ik is intruder with properties of initial position x, velocity vector v and ci,j,k is target cell.

(eq. E.3).

The space intersection rate for body volume PB(ik,ci,j,k) is defined as PT (ik(x,v,r),ci,j,k) (eq. E.7), where intruder ir has additional property of the intruder body volume radius r.

The space intersection probability for maneuverability uncertainty PS(ik,ci,j,k) is defined as PTD(ik(xs,v,θ,ϕ),ci,j,k) (eq. E.25), where intruder properties θ, ϕ stands for intruder horizontal and vertical uncertainty spread.

The time intersection rate Pτ,x(ik,ci,j,k) ∈ [0,1] is defined in (eq. ??). This probability has two calculation modes, first is for 1D intersection (line), second is for volume intersection (body volume, spread elliptic cone).

UAS cell entry time te and cell leave time tl time for a vehicle in avoidance grid A(ti) is given by (eq. ??) and (eq. ??).

Intruder leave and entry time for 1D intersections is trivial and is omitted in this section. Intruder entry ie and intruder leave il for 3D intersection is given by (eq. E.27, E.28).

All partial rates with respective definition references are summarized in (eq. E.29)

)	(E.3)

)	(E.7)

(E.29)

)	(E.25)

(??)

With definition of all space and time intersection rates (eq. E.29) and given flag vector l,b,s,τ ∈ {0,1} one can formulate combined intersection rate POI(ik,ci,j,k,l,b,s,τ) (eq. E.30) for intruder ik and cell ci,j,k. The principle is following: maximum of selected rates product based on flag vector is final intersection rate of intruder ik in the cell.

The time-use flag τ is adding time intersection rate Pτ,x(ik,ci,j,k), where time intersection rate is defined by x = {L,B,S} for line, body volume, spread ellipse time intersections (Pτ,L(ik,ci,j,k) 6= Pτ,B(ik,ci,j,k) =6 Pτ,B(ik,ci,j,k) for one intruder ik).

|  τ = 0 POI(ik,ci,j,k,l,b,s,τ) = τ = 1 |   PL(ik,ci,j,k).l   : max PB(ik,ci,j,k).b PS(ik,ci,j,k).s   P  τ,L(ik,ci,j,k).PL(ik,ci,j,k).l : max Pτ,B(ik,ci,j,k).PB(ik,ci,j,k).b Pτ,S(ik,ci,j,k).PS(ik,ci,j,k).s | (E.30) |
| --- | --- | --- |

Bibliography

11