6.3	Space Discretization - Avoidance Grid

Operation Space:	The Operation Space is a space where UAS can effectively surveillance its surroundings, and it has the capability to act.

The motivation for Discretization: The UAS surroundings needs to be represented in an avoidance-friendly manner, following principles matters:

- Discrete representation - the space around UAS should be segmented into finite and exclusive portions which are considered as one point of the grid. This enables fast situation assessment.

- Threat proximity - a threat in any form is getting more important with decreasing distance to UAS.

- LiDAR swipe density - one LiDAR swipe scans many points; the grid needs to be customized to swipe characteristics.

The Main Sensor is LiDAR (problems ??.-??). The effective occupancy computation needs to be done for all problems; the inspiration is taken from [1]. The effective occupancy computation is done in LiDAR scan portioned into polar coordinates grid. The operation space is abstracted as a grid where space portions are representing the points in the grid.

Note. Each member of the grid is a cell, represented as a point with shared properties, like threat level, visibility.

The Discrete Situation Evaluation is executed for a UAS local coordinate frame in fixed time. The goal is to enable fast discrete situation assessment.

LiDAR Swipe: The point scanned by LiDAR, where the UAS position is center of the local coordinate frame, and UAS heading is defining the main axes is given as:

point = [distance,horizontal◦,vertical◦].	(6.1)

Note. For polar/Euclidean transformations and local/global coordinate frames refer to background theory (app. ??).

The right side of UAS horizontal◦ ∈ ] − π,0[, the left side of UAS horizontal◦ ∈ [0,π], the downside of UAS vertical◦ ∈ ] − π,0[, the top side of UAS vertical◦ ∈ [0,π]

LiDAR Swipe Portioning: The polar coordinate space can be portioned into distinctive cells, which contains the portion space. This cell then represents one point in the grid.

2

The reason for this swipe portioning is LiDAR scanning density1, which is extremely dense. The threat state in the cell can be assessed with linear complexity.

The polar → euclidean coordinate frame transformation is not amenable for LiDAR swipe. The threat assessment based on LiDAR swipe in planar space portions has minimal complexity, and it is cost effective. [2].

Cell: To discretize operational space into a grid of points there is a need to define cell space, which bounds the portion of the local planar coordinate frame. The point (eq. 6.1) is defined by distance, horizontal◦ offset angle, and vertical◦ offset angle. The cell is a closed compact set of such points. The boundary can be defined like follow:

Definition 1. Cell

The cell bounds a portion of space in UAS local polar coordinate frame, defined by boundary ranges:

- Distance Range - starts and ends distancestart < distanceend in R+.

- Horizontal Range - starts and ends: .

- Vertical Range - starts and ends: by .

The space portion belonging to the cell is given by function as:

cell.spacePortion...

:



# 

To evaluate a static obstacle threat, it is necessary to know how many LiDAR hits landed in the cell space portion. For one LiDAR Scan the hits set is given a set of all points which lands into cell space portion:

cell.LiDARHits = {point ∈ LidarScan : point ∈ cell.spacePortion}	(6.3)

Note. The cell space portion volume is increasing with the distance. This satisfies the requirement for threat-distance importance. The cell is considered as a point of the grid with common properties abstraction valid for all cell space portion.

1Example rotary LiDAR Velodyne VL-16 specs: https://www.cadden.fr/wp-content/uploads/

2017/02/Velodyne_VLP-16-Puck.pdf

Effective Operation Space: The goal is to determine which of the operation space is going to be considered in our avoidance grid. The effective operation space determination according to [3] is influenced by the following factors:

- Sensors ranges - there is no reason to assess the situation over effective sensor range.

- Information sources impact - there is no real impact on effective space boundary, the information search and intersection algorithms are only of the importance.

- UAS maneuverability - the space where UAS can maneuver, bounded by space-time (reach set boundary).

- Computation power - the situation evaluation and threat assessment capabilities of the onboard computer.

- Airworthiness requirements - the regulations can impose some minimal requirements on effective operation space boundary.

Let show an example of an effective operation space for the UAS (fig. 6.1). The full LiDAR Swipe (cyan and red lines) of UAS (blue plane) has a shape of the conical cylinder.

Note. Under ideal circumstances, the LiDAR swipe would have a ball shape, but in real cases the UAS body portion where LiDAR is mounted is unused.

The frontal portion (red line) is a set of cells where UAS can make maneuvers. According to the previous conditions, there is no reason to consider a space portion out of the maneuverable area.

Figure 6.1: Example: The LiDAR reading portioning - cells.

4

Avoidance Grid Definition: The effective operation space is going to be portioned into cells. The set of these cells is going to be called Avoidance Grid. The idea is to split operational space into cells with even distance, horizontal angle, and vertical angle ranges.

Definition 2. Avoidance Grid

The effective space portion (fig. 6.1 red lines) given by a portion of space in UAS local polar coordinate frame, bounded by:

- Distance Range - in range distancestart < distanceend in R+.

- Horizontal Range - in range by.

- Vertical Range -in range .

The goal is to separate the effective operation space into cells (def. 1). The idea is to split distance range into multiple distinctive distance ranges with count layerCount ∈N+. The ranges for distance layers are given as follow:

;	i ∈ 1...layerCount	(6.4)

The same separation Layer horizontal/vertical separations defined by horizontalCount ∈N+/verticalCount ∈N+:

;	j ∈ 1...horizontalCount

(6.5)

;	k ∈ 1...verticalCount	(6.6)

Then celli,j,k space portion by (def. 1) has the following ranges:

- Cell Distance Range (eq. 6.4) depending on layer index i.

- Cell Horizontal Angle Range (eq. 6.5) depending on horizontal angle index j.

- Cell Vertical Angle Range (eq. 6.6) depending on vertical index k.

Note. The example of Avoidance Grid Cells is given in (fig. 6.1 red boundary).

The Avoidance Grid is the set of cells:

	 i ∈ 1...layerCount

# 	

AvoidanceGrid =	celli,j,k : j ∈ 1...horizontalCount	(6.7) 	k ∈ 1...verticalCount 

Note. For any distinctive cells celli,j,k, cellm,n,o their space portion intersection is empty set:

∀celli,j,k,cellm,n,o : celli,j,k ∩ cellm,n,o = ∅,i 6= o ∨ j 6= n ∨ k 6= o	(6.8)

Grid Sizing Approach:	The sizing approach used in this work is outlined in (app.

??).

Cell in Avoidance Grid Properties: For each cell p~ ∈R3 in the there are properties to be checked:

- Is there visibility to the cell? - how good is an observation of the cell by Sensor

Field.

- Is there threat present ? - how sure the data fusion is that there is eminent threat in the cell.

- Is the cell reachable? - if there is any trajectory which can get UAS to that cell without too much threat along the way.

The answers to these questions are given later in data fusion procedure outline (tab. ??). 6

# Bibliography

- Florian Homm, Nico Kaempchen, Jeff Ota, and Darius Burschka. Efficient occupancy grid computation on the gpu with lidar and radar for road boundary detection. In Intelligent Vehicles Symposium (IV), 2010 IEEE, pages 1006–1013. IEEE, 2010.

- Sandeep Gupta, Holger Weinacker, and Barbara Koch. Comparative analysis of clustering-based approaches for 3-d single tree detection using airborne fullwave lidar data. Remote Sensing, 2(4):968–989, 2010.

- Osmar R Za¨ıane and Chi-Hoon Lee. Clustering spatial data when facing physical constraints. In Data Mining, 2002. ICDM 2003. Proceedings. 2002 IEEE International Conference on, pages 737–740. IEEE, 2002.

7