6.4	Reach Set Approximation

Motivation: Reach set is a strong tool for Obstacle Avoidance because it contains all possible avoidance maneuvers. The current implementations (sec. ??) have the following flaws:

- Realistic approximation - nonlinear systems or heavily constrained systems cannot be approximated well by linear continuous-time Reach Sets.

- Finite count of possibilities - continuous-time Reach Set contains infinite possibilities for avoidance maneuvers; the SAA system demands conflict resolution infinite time.

- Computationally feasible data structures - binding related properties seem problematic because continuous- time reach sets do not have a unique identifier of maneuver, trajectory nor segment.

Proposed Solution Features: Our Reach set Estimation method will provide the following features:

- System Control Interface - implemented via Movement Automaton, requiring only a discrete command chain to approximate system behavior.

- Finite count of possibilities - finite number of elements in Reach set will enable scalable calculation.

- Computationally feasible data structures - approximation of Reach set as a set of trajectories, each trajectory can be split into a finite number of segments. Each element will have a unique identifier enabling both-side property binding.

- Computationally feasible data-structures - some specific behavior, like horizontal/vertical separation, or maneuver shape can be encoded into different types of reach set approximation algorithms.

# 6.4.1	Trajectory Set Approximation of Reach Set

Discretization of Reach set: There is a need for a discrete finite Reach Set approximation to enable Avoidance Strategy Evaluation infinite time. Replacing Continuous Control Set Inputs(t) by Movement Automaton is feasible:

Definition 1 (Reach set Approximation by Movement Automaton). A trajectory (def. ??) for system state˙ = f(time,state,input) under control of the movement automaton

MA is given as execution of movement buffer (def. ??) with an initial state of system state0.

Therefore notation Trajectory(state0,buffer) is used.

The Complete Reach Set (6.1) for system with initial state state0 with existing control strategy control(time) ∈ Controls(time). for time τ > time0.

ReachSet(τ,time0,state0) = [{state(s) : control(s) ∈ Controls(s),s ∈ (time0,τ]}

(6.1)

The Reach Set Approximation by Movement Automaton (6.2) of the system under the control of the movement automation MA consist from the set of trajectories Trajectory (state0, Buffer), which are executed in constrained time τ > time0.

	 duration(buffer)

	

ReachSet(τ,time0,state0) =	Trajectory(state0,buffer) :	≤	(6.2)

	(time0 − τ) 

Note. Reach Set Approximation (def. 1) is a subset of Full Reach Set (def. ??) in continuous space Rn it inherits all important properties, like Invariance [1].

Discretization of Reach Set have been achieved leaving us with a finite count of Trajectories, instead of Infinite subspace or RN

Approximated Reach Set Containment: The Approximated Reach Set introduced in (def. 1) is constrained only by future expansion time τ. UAS makes space assessment in Avoidance Grid. There is no point to consider Trajectories outside of Avoidance Grid

Definition 2 (Contained Approximated Reach Set). For a pair (state0, AvoidanceGrid0) at time time0 and prediction horizon τ = ∞ there is Contained Reduced Reach Set:

  	 time0, Trajectory(...)

   ∀segment ∈ AvoidanceGrid0, ReachSet state0,  = ∈ :

	  ReachSet(6.2)	segment ∈ Trajectory(...) 

AvoidanceGrid0

(6.3)

Properties: Container Approximated Reach Set contains only trajectories where all segments belong to Avoidance Grid, there are following functions:

- Membership function for any Trajectory in Constrained Reduced Reach set returns Ordered Set of Passing Cells.

- The cost function for any Trajectory Portion in Constrained Reduced Reach Set return Cost of Execution

Passing cell:	Cell of Avoidance Grid which has some intersection with Trajectory.

Note. It contained Reduced Reach Set (eq. 6.3) which is contained in the Avoidance Grid and have a Membership Function enable Property transition between Reach set and Avoidance grid.

Example: Visibility from cells along Trajectory can be gathered to calculate Trajectory‘s feasibility.

Reach Set Pruning: There is a need to implement Set Difference between Reach Set and Constraint Set. Constraint Set can be an Obstacle Set from Known World (sec. ??) and other different constraints.

Reach Set Trajectory Tree: (6.4) Any Reach Set where Control Strategy Constraint is implemented as Movement Automaton, with defined Movements set and for single initial state0. The Reach Set is given as discrete tree with root Trajectory(state0,∅).

For each Trajectory Segment, there exists intersection function which evaluates as true if there exists at least one point in Segment which belongs to Constraint Set. Formally:



∃point ∈ segment,

	: true

intersection(segment,Set) :	point ∈ Set	(6.5) Otherwise	: false

Definition 3 (Pruned Reach Set). For Reach set represented as Trajectory Tree (eq. 6.4) and some constraint set (Set) where exist intersection function (eq. 6.5). The Pruned Reach set is given as follows:

Note. Pruning(def. 3) [2] is applied multiple times for various Constraints Set.

Example of Approximated Reach set Calculation (def. 1), Reach Set Containment (def. 2), and, Pruning is given in [3].

# 6.4.2	Distinctive Properties of the Trajectories

Motivation: The need to Make Reach Set scalable approach. This may be a problem due to the Expansion rate. Reach set represented as a Trajectory Tree (eq. 6.4) for Avoidance Grid with layer count and Movement automaton with movement count, the Node count is given as:

(6.7)

This scaling is not feasible for Avoidance Grid with many layers (< 10) or Movement Set with many movements (< 9). There is a need for Reduced Reach set calculation.

Performance Criteria: The scaling factor (eq. 6.7) shows that there are going to be many trajectories. The main point is that not every trajectory in Reach Set is giving us maneuverability advantage. Our expectations lie in following Performance Requirements:

- Reach set must Cover maximum of the possible unique maneuvers in Avoidance

Grid.

- Trajectories in Reach Set should be smoothest possible to prevent cargo damage / UAS wear.

Trajectory footprint: Discrete space of Avoidance Grid is organized in cells. The cell is a minimal space portion accessible by property binding. There is a need to know if two trajectories contribution to Maneuverability in this environment.

Each trajectory passes through space in Avoidance Grid. If there exists a method to extract unique identifier for each trajectory passed cells, we can compare two trajectories Coverage in Avoidance Grid.

Definition 4 (Trajectory footprint). For Trajectory from Reach set (def. 2) defined for Avoidance Grid has membership function. Membership Function returns ordered set of passing cells:

(6.8)

Then we can define equality function for Trajectory1 and Trajectory2, as the comparison of their footprints in common Avoidance Grid as follow:

(6.9)

Note. Depending on Movement Automaton‘s movement set and Avoidance Grid parameters, there can be multiple trajectories which are equal.

Coverage set: Now it is possible to create a set of unique trajectory footprints due to footprint function (eq. 6.8). Similarly, there is a possibility to create Reach set skeleton containing unique trajectories, by using equality function (eq. 6.9). Coverage set is sufficient for now.

Definition 5 (Coverage Set). Coverage set (6.10) is defined for Avoidance Grid and Reach Set pair as a set of unique Trajectory footprints:

! (	 	!	)

AvoidanceGrid,	Trajectory,	∀Trajectory

coverages	=	footprint	:

ReachSet	AvoidanceGrid	∈ ReachSet

(6.10)

Coverage set properties: Trajectory footprint (eq. 6.8)is not a bijection, neither injection for ReachSet → CoverageSet. This implies the following properties:

- Equal Reach Sets in same Avoidance Grid have equal Coverage Sets.

- Equal Coverage Sets does not imply Reach Set equality.

- For two Coverage Sets, there is a possibility to compare their member count to create coverage ratio.

The second Property gives us a proposition that there is a possibility of Reach Set Reduction without losing Coverage.

Definition 6 (Coverage Ratio). Coverage Ratio is a ratio of Coverage Set Member Count between two Reach Sets. Reach set with a lesser count of unique Trajectories is considered as Reduced Reach Set. Reach set with greater Count of unique Trajectories is considered as Reference Reach Set.

Note. Reference Reach Set is usually Full Reach Set containing all possible trajectories in space contained by Avoidance Grid. In case Full Reach Set cannot be computed, Avoidance Grid is too large, most complex Reach Set is used as Reference Reach Set.

Trajectory smoothness:	Trajectory other than straight line have some changes in UAS heading.

The goal is to minimize Maneuvering of UAS, because:

- Every Heading Change needs to be reported to UTM.

- Sharp Maneuvering can damage cargo/wear UAS.

- Often course changes make Intruder prediction harder for other Civil General Aviation.

For this purpose, Smoothness Metric needs to be applied for Reach Set or Trajectory. In the case of Movement Automaton Control, two distinguish Movement Sets are introduced: Smooth and Chaotic movements set with the following properties:

MovementSet = SmoothMovements ∪ ChaoticMovements

SmoothMovements ∩ ChaoticMovements = ∅	(6.12)

|SmoothMovements| > 0,	|ChaoticMovements| > 0

Then Smoothnes clasificator for Trajectory(initialState,buffer) can be defined as isSmooth and Smooth Movement Counter function as smoothCount like follow:



movement ∈ SmoothMovements	: 1

isSmooth(movement) =

movement ∈ ChaoticMovements	: 0

(6.13)

X

isSmooth(movement),

smoothCount(Trajectory(...,buffer)) =

∀movement ∈ Buffer

Definition 7 (Smoothness Rating for Trajectory). Smoothness for trajectory generated by Movement Automaton for some Initial State with some Movement Buffer, under the assumption of Smooth and Chaotic Movement Set split (eq. 6.12), with existing classification and counter functionals (eq. 6.13) is given as follows:

For Trajectory with buffer = ∅ Smoothness is given as 1.

# 6.4.3	Heuristic Trajectory Tree Building

Motivation:	Purpose of Navigation is to move forward to Goal Waypoint in Mission.

Structure of Avoidance Grid is designed to enable forward and turning maneuvers. The

Avoidance Grid is organized in Layers characteristic by the same distance from Avoidance Grid Origin.

Survey of motion planning algorithm was given in [4]. The ideal candidate for propagation algorithm is Wave-front algorithm propagating Trajectory tree through Layers. Due to the Avoidance Grid onion-like layers, there is a possibility to implement turn maneuver through layers iterative and effectively.

Figure 6.1: Rapid Exploration tree as a result of Constrained trajectory expansion.

Rapid Exploration Tree (fig. 6.1) was selected because it enables Movement Automaton Utilization and Property Binding. A similar approach was used for space exploration [5].

The example (fig. 6.1) shows a Rapid Exploration Tree in Free Space containing Waypoint Navigation Path and Turn Away Path. Both paths are starting in same Root Node (red circle) which was expanded with simple Movement Automaton (a bunch of nodes originating from one node is showing the way of expansion). The connection (blue line) between two nodes (red circles) represents Trajectory portion for Executed Movement.

Rapid Exploration Tree Node	will contain the following information:

- Initial state - root entry point, used in state evolution calculation.

- Trajectory (state evolution) - trajectory passing through state space in the local coordinate frame of Avoidance Grid.

- Buffer (applied movements) - ordered list of executed movements applied on the initial state to obtain state evolution.

- Cost - calculated for state evolution based on a predefined cost function.

- Footprint - ordered set of passing cells in Avoidance Grid.

- Parent Node Reference - tree reference for the parent node, not in case of the root node.

- Other Bounded Properties - value list of other properties, depending on Expansion Constraints and Reachability evaluation algorithm.

Wave-front propagation of Rapid Exploration Tree	is given in (alg. 6.1).

The Avoidance Grid have UAS with position ∈ Initial State at the origin. The Grid Layer is a column ordered the set of cells with same Mean distance from the origin. Grid Layers are indexed from origin starting with 1; there is a maximum of i ≥ 1 layers.

Step: Initialization	contains base structure preparation like follows:

- Avoidance Grid - Space was containing Reach set (def. 2).

- Movement Automaton - Used as Predictor, consuming buffer containing Movements to generate Trajectory(initialState, buffer).

- Reach Set - tree consisting from Wave-frontNodes representing the endpoint of Trajectory(initialState, buffer) where each Edge represents one Movement application. The root is set as a node containing Initial State.

Function initializeReachSet(root, stack, grid, automaton) will take root and enforces full wavefront propagation to First Layer.

Step: Wave-front Propagation is forced propagation of trajectories from layer i to layer i + 1. The process goes as follows:

- Selection of Feasible candidates - function [candidates,leftovers] = ExpansionConstraints.select(stack) for working layer, row and cell selects feasible trajectory nodes ordered by Cost function. The Example of Cost Function can be Trajectory Smoothness (def. 7).

- Expansion of Candidates - for each candidate function candidate.expandNode (automaton) is invoked. This function will expand Candidate Node structure by appending Full Trajectory Tree Evolution until each Leaf Trajectory reaches Next Layer. Simply put Par rent Node Node(initialState, buffer, cost, footprint ) buffer is appended by movements until the next layer is reached.

- Leftovers purge - function reachSet.purge(leftovers) removes unexpanded Nodes leading to cell, effectively removing trajectories which do not lead to the next layer.

- Append Reach Set - function reachSet.append(leafs) puts newly created Nodes (Trees) into Reach Set structure. The Wave-front Propagation for one cell is finished.

Step: After Layer Propagation Purge is covered by function reachSet.purgeSameFootprint() which takes trajectories with the same footprint and keeps some of them based on Selection criteria, more in (sec. 6.4.4, 6.4.5). Pruning methods over Large Decision Trees are fast and viable [6].

Note. Reach Set is usually computed Prior the Flight for some Initial State in Local Coordinate Frame in right had coordinate frame with X+ used as main axis.

Algorithm 6.1: Wave-front propagation of Rapid Exploration Tree to form

Reach Set.

Input : Node(initialState,buffer=∅,cost=0,footprint=∅), AvoidanceGrid,

ExpansionConstraints, MovementAutomaton(movementSet) Output: ReachSet(AvoidanceGrid)

# Initialization Sequence; grid=AvoidanceGrid, automaton=MovementAutomaton, root = Node; reachSet = initializeReachSet(root,stack,grid,automaton);

# Main Expansion through, layers (i), rows (j), cells(k);

for

layer(

1

...i

)

in

grid

do

for

row(

1

...j

in

layer)

do

for

cell(

1

...k

)

in

row

do

#applyselectioncriteria;

[

candidates,leftovers]=ExpansionConstraints.select(stack);

#collectexpansions;

leafs=[];

for

candidate

in

Candidates

do

leafs=[leafs,candidate.expandNode(automaton)];

end

reachSet.purge(leftovers);

reachSet.append(leafs);

end

end

reachSet.purgeSameFootprint();

end

# 6.4.4	Coverage-Maximizing Reach Set Aproximation

Motivation: Design of calculation method for Reach Set Approximation guarantying high Maneuverability.

Background: There is Coverage Ratio property of Reach Set (def. 6). It has been shown that creating Reach Set via greedy approach is not feasible due the Scaling Factor. Contracted Expansion (sec. 6.4.3) is enabling to apply selection criteria while building Reach Set in given Cell.

The Cell celli,j,k has a center and walls from UAS viewpoint: a front wall, back wall (for layer > 1), a top wall, left wall, right wall, bottom wall. It is expected that trajectory leading close to one cell walls will continue to a different cell, increasing the chance to obtain more Unique Footprints.

Expansion Constraint Function Implementation (alg. 6.2) is based on the simple principle: Select candidate Nodes which are closest to outer walls of Cell, with a unique footprint.

Tuning Parameters: Proximity to Cell outer wall gives good chances to break into other rows or columns in the Avoidance Grid. Unique footprint guarantees future Unique Footprint after appending Trajectory by Movement application.

- Considered Footprint Length - how much last cells in footprint should be considered in unique path track, minimal value 1, default value 3, maximal value ∞. If you want to generate nonredundant trajectories use ∞, it will consider full footprint.

- Spread Limit - the upper limit of candidates which are going to be select for further expansion, minimal value 1, default value Count of unique Moves in Movement set, maximal value ∞. If more than default values are selected, the algorithm will generate redundant trajectories. If less is selected, then some trajectories are omitted, and Coverage Rate decreases sharply.

Step: Initialization initialization of candidate array (return value), leftovers array (return Value). Node array passing is populated with Nodes which represents end node of Trajectory, and the tip of the trajectory is constrained in the cell,j,k.

Step: Evaluate best trajectories with unique Footprints following steps are executed:

- Best Performance Map is created with a footprint as a key set element to ensure footprint uniqueness.

- Wall distance for the test node is calculated as a closest trajectory portion distance to the top, bottom, left, right wall of cell cell,j,k

- The footprint for the test node is created with the maximal length given by Footprint Length tuning parameter.

- Existence and Performance Test is executed to ensure that the best performing node is selected. If there is no key entry in the Best Performance Map, then a new entry for Test Node is created. If there is a key entry, the performance of Old Node and Test Node is compared, and better is stored.

Step: Select candidates is executed on Best Performance Map records using Wall distance as pivot parameter, ordering by closest proximity and limited by Search Limit tuning parameter. The Leftovers are difference set between Passing Nodes and Candidate Nodes.

Algorithm 6.2: Expansion Constraint function for Coverage-Maximizing Reach

Set Approximation

Input	: Node[] stack, Cell celli,j,k

Tuning Parameters: int+ footprintLength, int+ spreadLimit Output : Node[] candidates, Node[] leftovers

# Initialize structures;

Node[] candidates = [], Node[] leftovers=[];

Node[] passing = celli,j,k.getFinishingTrajectories(stack);

# Select best performing trajectories with unique footprint; Map<Footprint,Node> bestPerformanceMap; for Node test ∈ passing do

else

end

wallDistance= test.minimalDistanceToWall(celli,j,k)]; footPrint = test.getFootprint(lastCells = footprintLength); if bestPerformanceMap.contains(footPrint) then old = bestPerformanceMap.getByKey(footprint); oldPerformance= old.minimalDistanceToWall(celli,j,k); if oldPerformance > wallDistance then bestPerformanceMap.setByKey(footprint,test); end

bestPerformanceMap.setByKey(footprint,test);

end

# Select best performing nodes up to spreadLimit count; candidates = bestPerformanceMap.select(count =

spreadLimit).orderBy(’wallDistance’,’Ascending’);

leftovers = passing - candidates; return [candidates,leftovers]

Example:	for Avoidance Grid with Distance 10 m, Layer count 10, Horizontal range

], Horizontal Cell Count 7, Vertical range ], and Vertical Cell Count 5. Is given in (fig. 6.2). The UAS is at Back-side of Figure (initial state is at all Trajectory Origins). The black dashed line marks Avoidance Grid space boundary. Each trajectory has its own color and ends at Front-side of Avoidance Grid Boundary.

Figure 6.2: Coverage-Maximizing reach set approximation.

Pros and Cons: It can be seen from example (fig. 6.2) that Coverage-Maximizing Reach Set Approximation Method (alg. 6.2) generates much turning and shaky trajectories.

High Coverage Ratio (∼ 0.9) is provided while keeping medium node count. The calculation complexity scales linearly with grid size. The upper limit of trajectories is given as follow:

countTrajectories(ReachSet) ≤ layerCellCount × spreadLimit

× size(Movements)	(6.15)

The upper limit of nodes is given as follow:

countNodes(ReachSet) ≤ layerCount × layerCellCount

× size(Movements) × spread limit	(6.16)

The absence of Smooth Trajectories disqualifies Coverage Maximizing -RSA to be used for Navigation. This type of reach set is feasible for Avoidance because it contains a variety of maneuvers.

# 6.4.5	Turn-Minimizing Reach Set Approximation

Motivation:	Imagine having an Avoidance Grid like (fig. ??). There is a need of

Reach Set Approximation which will have Smooth Trajectories (def. 7) going nearby cell centers.

Background: The Smoothness Rating for Trajectory (def. 7) uses two distinct sets Smooth Movements and Chaotic Movements (eq. 6.12) which are defined for our Movement Automaton (sec. ??) like following:

SmoothMovements = {Straight}

(6.17)

ChaoticMovements = Movements − SmoothMovements

Smooth Movements contains only Straight movement because others are considered as extreme turning movements. Smooth Movements should contain only direct flight movements or slight heading correction. Chaotic Movements set is a supplement of Movement Automaton‘s Movement Set.

The Avoidance Grid (fig. ??) cell centers for fixed indexes jfix, kfix are linearly aligned with the initial state. That means that cell centers of cells cell1,jfix,kfix,...,celli,jfix,kfix, where i is a count of layers lie on one line. If the trajectory can achieve cell center on some layer, only minor trajectory corrections are required to stay on the given line. This type of trajectory gives us the following advantages:

- Minimal steering at the beginning - the minimal steering is advantageous in Controlled Airspace because is diminishing the amount of communication to UTM Service.

- Additional safe space in the Linear segment - once the center of the cell is reached, Trajectory sticks to the line between cell centers. Each point on this line has a maximal distance to outer walls of the cell. This gives us extra space given as minimum of distance between UAS position and Outer cell walls.

Expansion Constraint Function Implementation (alg. 6.3) is based on the simple principle: Select candidate Nodes which are closest to Cell center, with a unique footprint.

Note. Cell center can be closely reached by smooth movement from a previous cell or chaotic movement from a neighbouring cell from the current or previous layer. These trajectories are usually equivalent in Smoothness.

Tuning Parameter: Proximity to Cell Center gives a good chance to keep trajectory smooth or smooth after one correction maneuver. It has been mentioned that Cell Center can be reached by various trajectories. In this method full footprint length is always considered; therefore only one tuning parameter can be offered:

1. Spread Limit - the upper limit of candidates which are going to be selected for further expansion, minimal value 1, default value Count of unique Moves in Movement set, maximal value ∞. If the maximal value ∞ is selected, the algorithm will generate the skeleton of Reach Set with full Coverage and with the smoothest Trajectories.

Step: Initialization sets candidate Nodes as empty set, leftover Nodes as empty set. and selects all Nodes from Stack which represents Finishing Trajectories in working cell

celli,j,k.

Algorithm 6.3: Expansion Constraint function for Turn-Minimizing Reach Set

Approximation

Input	: Node[] stack, Cell celli,j,k

Tuning Parameters: int+ spreadLimit

Output	: Node[] candidates, Node[] leftovers

# Initialize structures;

Node[] candidates = [], Node[] leftovers=[];

Node[] passing = celli,j,k.getFinishingTrajectories(stack);

# Select unique smoothest trajectories; Map<Buffer,Node> bestPerformanceMap; for Node test ∈ passing do

else

end

centerDistance= test.getPerformance(celli,j,k)]; footPrint = test.getFootprint(); if bestPerformanceMap.contains(footPrint) then old = bestPerformanceMap.getByKey(footprint); oldPerformance= old.getPerformance(celli,j,k); if oldPerformance > centerDistance then bestPerformanceMap.setByKey(footprint,test); end

bestPerformanceMap.setByKey(footprint,test);

end

# Select best performing nodes up to spreadLimit count; candidates = bestPerformanceMap.select(count =

spreadLimit).orderBy(’cellCenterDistance’,’Ascending’);

leftovers = passing - candidates; return [candidates,leftovers]

Step: Evaluate smoothest trajectories with unique Footprints is implemented as multi-criteria filtration.

The first criterion is the distance to Cell Center which is penalized by trajectory smoothness rate implemented in method Node.getPerformance(Cell celli,j,k) defined as follow.

(6.18) Distance of Trajectory is enumerator because its considered as the base value and is defined in the interval [0,maximalWallDistance]. The Smoothness Rate is in the denominator, because it is a penalization coefficient defined in the interval [0,1].

The second criterion is trajectory uniqueness. This is provided by Best Performance Map, where best performing Node belongs to one unique trajectory footprint. The implementation is identical to coverage-maximizing set expansion (alg. 6.2).

Step: Select candidates is executed on Best Performance Map records using Penalized Cell Center Distance as pivot parameter, ordered in ascending order and limited by Spread Limit tuning parameter. The Leftovers are difference set between Passing Nodes and Candidate Nodes.

Example:	for Avoidance Grid with Distance 10 m, Layer count 10, Horizontal range

], Horizontal Cell Count 7, Vertical range ], and Vertical Cell Count 5. Is given in (fig. 6.3). The UAS is at Back-side of Figure (the initial state is at all Trajectory Origins). The black dashed line marks Avoidance Grid space boundary. Each trajectory has its color and ends at Front-side of Avoidance Grid Boundary. The Spread Limit, in this case, was set to 9 which is Size of the Movement Set.

Note. Please note Trajectories are organized in bundles going around Cell Centers smoothly. Most of the steering maneuvers are executed at the beginning of the Avoidance Grid.

Figure 6.3: Turn-minimizing reach set approximation.

Pros and Cons: It can be seen from example (fig. 6.3) that Turn-Minimizing Reach Set Approximation Method (alg. 6.3) generates smooth evenly spread trajectories.

High smoothness ratio (≥ 0.9) is provided while keeping low node count for UAS systems. The calculation complexity scales linearly with grid size. The upper limit of trajectories is given as follow:

countTrajectories(ReachSet) ≤ layerCellCount × spreadLimit

× size(Movements)	(6.19)

The upper limit of nodes is given as follow:

countNodes(ReachSet) ≤ layerCount × layerCellCount × spread limit	(6.20)

The absence of High Coverage Ratio disqualifies Turn-Minimizing Reach Set Approximation to be used for Emergency Avoidance. This type of Reach Set is feasible for Open Space Navigation or Controlled Airspace Navigation. Its low turning rate in contained Trajectories are desired for such tasks.

# 6.4.6	ACAS-X like Reach Set Approximation

Motivation: The implementation of ACAS-Xu behavior in DAA system will be mandatory for National Airspace System Integration in United spaces [7].

Implementation of ACAS-Xu like behavior increase usability of approach, if it can be achieved without major concept changes.

Background: The ACAS-Xu system on the operational level has been described in [8]. The Policy for Collision Avoidance proposal has been given in [9].

Some behavioral patterns can be encoded into Reach Set. ACAS-Xu navigation part is basically Look-up table of Maneuvers for Allowed Separations.

The Evasive Maneuver selection process in ACAS-Xu is similar to our approach: Select most energy efficient maneuver in compliance with space-time constraints. ACASXu intruder model is similar to our Body Volume Intersection Model (app. ??). The ACAS-Xu defines following base separations:

- Horizontal - movements on a Horizontal Plane in Global Coordinate System.

- Vertical - movements on Vertical Plane in Global Coordinate System.

There are allowed custom separations which can be used, for further experimentation:

- Slash - movement on +45◦ Tilted Plane to Horizontal Plane in Global Coordinate System.

- Backslash - movement on −45◦ Tilted Plane to Horizontal Plane in Global Coordinate System.

For given Movement Automaton implementation (sec. ??) the separations are given as follow:

Horizontal = {Straight,Left,Right}

V ertical = {Straight,Up,Down}

(6.21) Slash = {Straight,UpLeft,DownRight}

Backslash = {Straight,UpRight,DownLeft}

For each Node(...,buffer) and each separation there is a evaluation function isSeparation which decides, if Trajectory defined by node buffer is made up only from Separation movements. The function isSeparation(...) is defined like:



∀movement ∈ buffer,

	: true

isSeparation(buffer,separation) =	movement ∈ separation	(6.22)

otherwise	: false

Following Separation Modes can be defined with given separations:

- Horizontal (ACAS-X defined mode) containing horizontal separation.

- Vertical (ACAS-X defined mode) containing vertical separation.

- Horizontal-Vertical (ACAS-X defined mode) containing horizontal, vertical separations.

- Full (custom defined mode) containing all Separation Modes.

Note. Every separation modes generate 2D trajectories set on Respective plane. There is no need for Tuning parameters for further Expansion Constraint.

Expansion Constraint Function Implementation (alg. 6.4) is based on the simple principle: Select only candidate Nodes which Trajectories have at least one desired Separation Mode.

Step: Initialization sets candidate Nodes as the empty set, leftover Nodes as the empty set, and, select all nodes to form a stack which represents Finishing Trajectories in working celli,j,k,

Step: Candidate Selection Process is evaluated for each test Node from passing Node Set.

For each applicable separation, given as input parameter separations, The test function isSeparation (eq. 6.22) is applied:

- If test Node trajectory belongs to at least one allowed separation it is added to candidates set.

- Else is added to Leftovers.

Note. Separation sets (eq. 6.21) are not exclusive sets in Movement Automaton domain.

One Trajectory contained by Node can belong to multiple Separations.

Algorithm 6.4: Expansion Constraint function for ACAS-like Reach Set Approximation

Input	: Node[] stack, Cell celli,j,k, Separation[] separations

Tuning Parameters: None : ∅

Output	: Node[] candidates, Node[] leftovers

# Initialize structures;

Node[] candidates = [], Node[] leftovers=[];

Node[] passing = celli,j,k.getFinishingTrajectories(stack);

end

end

# Select nodes containing trajectories with usable separations; for Node test ∈ passing do for separation ∈ separations do

# Get separations for Node;

Separations[] nodeSeparations = test.getSeparations();

# If trajectory given by buffer is on Separation plane; if isIn(isSeparation(test.buffer,separation)(6.22) then candidates.append(test); end

# If there was no applicable separation, throw Node away; if test 6∈ candidates then leftovers.append(test);

end

# Return results; return [candidates,leftovers]

Example:	for Avoidance Grid with Distance 10 m, Layer count 10, Horizontal range

], Horizontal Cell Count 7, Vertical range ], and Vertical Cell Count 5. Is given in (fig. 6.4). The UAS is at Back-side of Figure (initial state is at all Trajectory Origins). The black dashed line marks Avoidance Grid space boundary. Each trajectory has its own color and ends at Front-side of Avoidance Grid Boundary.

Full separation mode is given in (fig. 6.4a). Horizontal-Vertical separation mode, used in original ACAS-Xu testing [8], given in (fig. 6.4b). Horizontal separation mode given in (fig. 6.4c) is usually used by planes. Vertical separation mode given in (fig. 6.4d) is usually used by copters.

(a) Full.	(b) Horizontal-Vertical.

(c) Horizontal.	(d) Vertical.

Figure 6.4: ACAS-X imitation reach set approximation for various separation modes.

Pros and Cons: It can be seen from examples (fig. 6.4) that ACAS-like Reach Set Approximation Method (alg. 6.4) generates a full reach set for 2D plane located in 3D space.

The Reach Set contains trajectories with high coverage ratio and high smoothness rating for selected 2D separation plane. Overall performance compared to full 3D reach sets (sec. 6.4.4, 6.4.5 6.4.7) is poor.

The node and trajectory count boundary was not implemented. It is common knowledge that 2D avoidance sets do not require scaling [8]. Otherwise, trajectory footprint mechanism like in Turn-Minimizing Reach Set Approximation (alg. 6.3) can be introduced.

This reach set implements Planar-Separation as a native feature, it can be used for both navigation and avoidance tasks in Controlled Airspace. For Non-controlled Airspace, there are far more superior Combined Reach Set (sec. 6.4.7).

# 6.4.7	Combined Reach Set Approximation - Tree Merge

Motivation: Turn-Minimizing Reach Set Approximation (sec. 6.4.5) is efficient for Navigation in Controlled Airspace. Coverage-Maximizing Reach Set Approximation (sec. 6.4.4) is good for Emergency avoidance. The need for the differentiation between Navigation and Emergency Avoidance mode is necessary for Controlled Airspace. But not in Noncontrolled Airspace. The combination of Turning-Minimizing and Coverage-Maximizing reach set approximations is an obvious solution.

Automatic mode switch can be provided by a combination of Navigation Reach Set and Avoidance Reach Set with an elevated cost function. Overall having a method to merge multiple trees would be beneficial.

Background: If two Reach Set Approximation were calculated for the same Avoidance Grid and Initial State, using same Movement Automaton and UAS model are possible to merge.

The Reach Set Approximation is a tree with Root Node in initial state with movement buffer = ∅. The movement buffer in each node can be used as route trace during the merging procedure. The example two reach set merge can be given as follow, where only the latest applied movement is taken into account.

	

First Reach Set

	

	left 

Combined Reach Set	

		

  left    →  left →  (6.23)

	∅



Second Reach Set

		

		→	

	*∅	 	right	right



 ∅ →	*left 



	right →	

right

First Reach Set contains two trajectories given by buffers {left,left} and {left,right}.

Second Reach Set contains two trajectories given by buffers {right,left} and {right,right}. The Combined Reach Set contains all four trajectories.

Note. The combined tree [10] does not need to have a combined amount of original Reach Sets trajectories. There can be Duplicity which means that any bounded property like Cost must be calculated again.

Combined Reach Set Calculation Function (alg. 6.5) is implemented as function NodecombinedReachSet(...) which takes root Node with initial State, Avoidance Grid and respective parameters for each calculation method. turn-minimizing spread for TurnMinimizing Reach set calculation and coverage spread, Footprint Length for CoverageMaximizing Reach Set Approximation.

Separate Reach Sets are calculated using Wave-front propagation (alg. 6.1) using respective Constrained Expansion functions for Turn-Minimizing (alg. 6.3) and CoverageMaximizing (alg. 6.2) reach sets.

Combined Reach Set is created using Node merge tree(...) function because different cost function or Bounded Parameters Calculation may be applied on Original Reach Sets.

Cost for each node needs to be recalculated due to original reach sets disparity. Function combined.applyCostFunction() will recalculate the new cost for each node.

The Goal is to have a penalization for Chaotic behavior, implementation of Automatic Mode Switch can be done like follows:

- Calculate Normal Cost for Node Cost(Node) for the associated trajectory:

Cost(Node.Trajectory).

- Calculate Penalization for additional maneuvering, calculate Smoothness Rating for Trajectory (def. 7) in the interval [0,1], introduce penalization with base 100%.

The final Cost(Node) function is applied to each Combined Reach Set Node and look like follows:

Cost(Node) = Cost(Node.Trajectory) × ...

··· × (1 + (1 − SmoothnessRate(Node.Trajectory)))	(6.24)

Tree Merge Function mergeTree(...) implements Outer Join operation on two trees. Example was given in (eq. 6.23). Function is applied on root Node iterating over Movements in Movement Set, because Movement is pivot.

Algorithm 6.5: Reach Set Merge Function and Combined Reach Set calculation

# Tree merge function;

Node mergeTree(Node firstNode, Node secondNode)

# Try to copy reference node or return null;

end

Node referenceNode = (firstNode?:(secondNode?: return null)); Node merged = new Node(referenceNode); merged.leafs= [];

# Try to fetch movement nodes if exist in any sub tree; for movement ∈ Movements do

firstLeaf = firstNode.getLeafFor(movement); secondLeaf = secondNode.getLeafFor(movement); newLeaf = mergeTree(firstLeaf,secondLeaf); if newLeaf ∼= null then

merged.leafs.append(newLeaf); end

return merged

# Combined Reach Set calculation function;

Node combinedReachSet(Node root, AvoidanceGrid grid,int+ coverageSpread, int+ turnSpread, int+ footprintLength)

Node cmrsa = chaoticReachSet(root,grid, footprintLength,coverageSpread);

Node tmrsa = harmonicReachSet(root,grid, turnSpread); Node combined = mergeTree(cmrsa,tmrsa); combined.applyCostFunction(); return combined

Example:	for Avoidance Grid with Distance 10 m, Layer count 10, Horizontal range

], Horizontal Cell Count 7, Vertical range ], and Vertical Cell Count 5. Is given in (fig. 6.5). The UAS is at Back-side of Figure (initial state is at all Trajectory Origins). The black dashed line marks Avoidance Grid space boundary. Each trajectory has its own color and ends at Front-side of Avoidance Grid Boundary. The Coverage-Maximizing Spread was set to 8, Footprint Length to 3 and Turn-Minimizing Spread to 1.

Note. Notice there are typical trajectories from both Turn-Minimizing (fig. 6.3) and Coverage-Maximizing (fig. 6.2) Reach Set Approximations.

Figure 6.5: Combined reach set approximation.

Pros and Cons: It can bee is seen from example (fig. 6.5) that Combined Reach Set Approximation (alg. 6.5) contains both types of maneuvers. Cheaper turn-minimizing for navigation and More Expensive Coverage-Maximizing for Emergency Avoidance. The upper limit of trajectories is given as follow:

countTrajectories(ReachSet) ≤ countTrajectories(CM − RSA)

+ countTrajectories(TM − RSA)	(6.25)

The upper limit of nodes is given as follow:

countNodes(ReachSet) ≤ countNodes(CM −RSA)+countNodes(TM −RSA) (6.26)

Turn-Minimizing Reach Set is ideal for Non-controlled Airspace missions, because it contains Automatic Mode Switch between Navigation and Emergency Avoidance.

24

# Bibliography

- Franco Blanchini. Set invariance in control. Automatica, 35(11):1747–1767, 1999.

- John Birmingham and Peter Kent. Tree-searching and tree-pruning techniques. In Computer chess compendium, pages 123–128. Springer, 1988.

- Alojz Gomola, Joa˜o Borges de Sousa, Fernando Lobo Pereira, and Pavel Klang. Obstacle avoidance framework based on reach sets. In Iberian Robotics conference, pages 768–779. Springer, 2017.

- Chad Goerzen, Zhaodan Kong, and Bernard Mettler. A survey of motion planning algorithms from the perspective of autonomous uav guidance. In Selected papers from the 2nd International Symposium on UAVs, Reno, Nevada, USA June 8–10, 2009, pages 65–100. Springer, 2009.

- Catherine Plaisant, Jesse Grosjean, and Benjamin B Bederson. Spacetree: Supporting exploration in large node link tree, design evolution and empirical evaluation. In Information Visualization, 2002. INFOVIS 2002. IEEE Symposium on, pages 57–64. IEEE, 2002.

- John Mingers. An empirical comparison of pruning methods for decision tree induction. Machine learning, 4(2):227–243, 1989.

- Jay Shively. Uas integration in the nas: Detect and avoid. 2018.

- Mike Marston and Gabe Baca. Acas-xu initial self-separation flight tests, 2015.

- Kyle D Julian, Jessica Lopez, Jeffrey S Brush, Michael P Owen, and Mykel J Kochenderfer. Policy compression for aircraft collision avoidance systems. In Digital

Avionics Systems Conference (DASC), 2016 IEEE/AIAA 35th, pages 1–10. IEEE, 2016.

- Patrick O’Neil, Edward Cheng, Dieter Gawlick, and Elizabeth O’Neil. The logstructured merge-tree (lsm-tree). Acta Informatica, 33(4):351–385, 1996.

25