7.6	Reduced Reach Sets Performance

Constrained Expansion Method (alg. ??) is creating Reach Sets from the Root Node as a tree expansion using Expansion Constraint function (depending on type).

The Reach set creation procedure is creating the following artifacts:

- Nodes - tree Node containing necessary data for discrete Trajectory portion, notably System State Evolution, buffer, and, Reachability Rating.

- Trajectories - leaf Node containing unique buffer which is not prefixed in others Node buffer.

The Reach Set Computation Time depends strongly on Movement Automaton prediction complexity and Node count. The Constrained Expansion Method (alg. ??) is separating all nodes entering into celli,j,k into two distinctive groups: Candidates for expansion and Leftover Nodes.

The Leftover Nodes are thrown away every expansion. The Leftover Nodes are not expanded in the next Wave-front iteration, but they leave a notable computation and memory footprint.

Note. Average Trajectory Smoothness Rate (def. ??) is important only in Navigation Mode; this aspect has been covered over (sec. ??, ??, ??).

Approach: For the same conditions (Testing Avoidance Grid, UAS initial state, Movement Automaton) compare the performance of Reach Set Approximations created by various methods for the following parameters:

- Coverage Ratio - defined in (def. ??) shows how versatile Reach Set Approximation is (up to 100% of complete reach set coverage).

- Node count - count of Nodes in Reach Set Approximation counted like:

  - full - all active nodes existing over computation time,

  - pruned - active nodes for real-time use.

- Count of Trajectories - count of Trajectories (leaf Nodes) counted like:

  - full - all active trajectories existing over computation time,

  - pruned - active trajectories are leading to coating cells of Avoidance Grid.

Testing Avoidance Grid	with Distance 10 m, Layer count 10, Horizontal range

], Horizontal Cell Count 7, Vertical range ], and Vertical Cell Count 5.

Note. The sizing of the Avoidance Grid was chosen the small scale because the property of Coverage Ratio can be calculated exactly up to some scale, after that it can be only assumed.

Various sizes of Avoidance Grid was tested in [1].

2

The UAS is at Back-side of Figure (an initial state is at all Trajectory Origins). The black dashed line marks Avoidance Grid space boundary. Each trajectory has its color and ends at Front-side of Avoidance Grid Boundary.

Chaotic Reach Set	(sec.	??) is used in Emergency Avoidance Mode for Non-

Controlled Airspace. The full set of trajectories is given in (fig. 7.1a). The Pruned set of trajectories is given in (fig. 7.1b).

Tuning parameters were selected like follow: Spread Ratio is 15 (unique footprint trajectories in the cell), and trajectory footprint length is 3 (last three unique passing cells).

(a) Full.	(b) Pruned.

Figure 7.1: Chaotic reach set computation example.

Harmonic Reach Set (sec. ??) is used in Navigation Mode for Non Controlled Airspace. The full set of trajectories is given in (fig. 7.2a). The Pruned set of trajectories is given in (fig. 7.2b).

Tuning parameter for harmonic spread ratio was set to 9 (which implies high coverage).

(a) Full.	(b) Pruned.

Figure 7.2: Harmonic reach set computation example.

Combined Reach Set (sec. ??) is combination of Chaotic Reach Set (fig. 7.1) and Harmonic Reach Set (fig. 7.2). The tuning parameters are the same for the respective methods.

It is used for both Emergency Avoidance and Navigation.

ACAS-like Reach Set (sec. ??) is used in Navigation Mode for Controlled Airspace. The separations used are Horizontal, Vertical, Slash, and, Backslash, to give the worst possible nodes and trajectories count. The full set of trajectories is given in (fig. 7.3a). The Pruned set of trajectories is given in (fig. 7.3b).

(a) Full.	(b) Pruned.

Figure 7.3: ACAS-like reach set computation example.

Computation Methods Performance Comparison (tab. 7.1) gives overview of memory consumption and Coverage Ratio.

Node count: Full Node Count shows how much memory it takes to compute Reach set. Pruned Node Count shows how much memory is needed for storage.

Note. The total size of full/pruned Reach Set depends on Node implementation. The Object-oriented prototype implementation in Matlab for example avoidance grid took up to 1 megabyte of system memory. The effective implementation would take up to 100 kilobytes.

Constrained expansion (alg. ??) have different selection rate, depending on the method. The survival rate directly reflects strictness of selection criteria. The rate of node pruned is summarized in (eq.7.1)

Nodes pruned

chaotic	: 78.93%

(7.1)

harmonic	: 18.50%

ACAS − like	: 79.05%

The interpretation or results for each reach set estimation method is like follow:

- Chaotic - the main exploration drive is Coverage Rate, the Trajectory segments are not usually smooth. For our Movement Automaton, there is only one Smooth Movement: Straight. Other eight are considered Chaotic Movements. Impact of this fact is significant because 4/5 of nodes were pruned.

4

- Harmonic - the main exploration drive is Smoothness of contained Trajectories. The Trajectory segments which are getting further away from cell center are not feasible. If Smooth Movements set size is considered, the Smooth/Chaotic ratio is 1/8 for our Movement Automaton implementation. The low node count was expected in this approach. Another Contributing factor is Trajectory Footprint Length for uniqueness selection, which is not a tuning parameter in this method, and it is set to the most strict selection.

- ACAS-like - the main drive is to create set consisting from multiple 2D separation planes. The expansion method applies full movement set on the candidate node. The Separation plane movement subset is determining, which node will be selected for further expansion. The size of the separation plane subset to the size of movement set rate is 1 : 3. There are four separation planes: horizontal, vertical, slash and backslash each containing full 2D plane reach set approximation which caused high node prune rate. Nodes used rate should get lower with increasing grid size.

Trajectories count: Full trajectories count shows how many leaf nodes were existing during the calculation process without pruning. The difference between full node count and full trajectories count is count of inner tree nodes. Pruned trajectories count shows how many leaf nodes are used in run-time of avoidance algorithm. The difference between pruned node count and pruned trajectories count shows the count of inner nodes in active reach set.

The most of waste leaf nodes are removed during layer pruning: function reachSet.purge- SameFootprint() (alg. ??). The Waste trajectories or unused leaf nodes count have significant impact. Because leaf nodes are a side product of Node Expansion procedure the amount of pruned trajectories is around 90 % regardless of the used method. The results are summarized in (eq. 7.2)

Trajectories pruned

| chaotic |  |  |  | : 91.24% |  | (7.2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| harmonic |  |  |  | : 88.21% |  |  |  |
| ACAS − like |  |  |  | : 89.43% |  |  |  |
| Calculation method | Node count |  |  | Trajectories |  | Coverage ratio |  | Parameters |  |
|  | full | pruned |  | full | pruned |  |  |  |  |
| chaotic | 6727 | 1417 |  | 4557 | 399 | 90% |  | spread:15 |  |
| harmonic | 1724 | 1405 |  | 1528 | 180 | 30% |  | spread:9 |  |
| combined | - | 2405 |  | - | 435 | 95% |  | CH spread:15 H spread:9 tree comb. |  |
| ACAS-like | 11294 | 2366 |  | 7437 | 786 | 74.95% |  | Separations: H/V/S/BS Coverage pruning: disabled |  |

Table 7.1: Reduced reach set computation methods performance

Coverage ratio: (def. ??) is showing how much maneuvering versatility of Reach Set. Full Reach Set Approximation have coverage ratio of 100 %. It is possible to construct Reference Reach Set without constrained expansion method which contains all possible trajectory footprints. Following observations for coverage ratio can be made:

- Chaotic reach set estimation method by design select Nodes which have the probability of trajectory footprint diversification. The high coverage ratio was achieved at values around 90 %.

- Harmonic reach set estimation method by design selects most smooth trajectories which cause low trajectory footprint diversity. The fairly high coverage ratio of 30 % has been achieved.

- Combined reach set estimation method takes two reach set and combines their trajectory trees into a single trajectory tree. It is given that Coverage ratio will achieve at least maximal coverage ratio of original reach sets. Harmonic reach set supplemented narrow smooth trajectories which were throw away previously — this increased overall coverage ratio to 95 %.

- ACAS-like reach set estimation method contained four separation planes, which caused that it was similar to Chaotic Reach Set Approximation for given Avoidance Grid, concerning performance. The coverage ratio For 2D plane was 100 %.

6

# Bibliography

[1] Alojz Gomola, Pavel Klang, and Jan Ludvik. Probabilistic approach in data fusion for obstacle avoidance framework based on reach sets. In Internal publication collection, pages 1–93. Honeywell, 2017.

7